QMS Quick Learning Activity

# Controls and Control Charting

# Quiz-Answer Key

Passing score = 80%   [Answers in RED]

Select the correct answer.

| 1. | T | F | The ISO/IEC 17025 Standard requires that the laboratory have procedures for monitoring the quality of test results. (Answer: T) |
| --- | --- | --- | --- |
| 2. | T | F | A laboratory may not use the same QC over and over again to create a control chart. (Answer: F) |
| 3. | T | F | An optimal QC sample is one that is the same, or similar, matrix as the test materials and has a value of the analyte close in range to the expected results. (Answer: T) |
| 4. | T | F | The mean value is used to calculate the upper and lower control limits. (Answer: T) |
| 5. | T | F | The x-axis on a control chart is used to delineate time. (Answer: T) |
| 6. 7. 8. 9. | T T T T | F F F F | When creating a control chart mean using a spiked sample, only one analysts should be used. (Answer: F) The upper control limit is calculated by multiplying the standard deviation by three and adding this result to the mean. (Answer: T) The lower warning limit is calculated by multiplying the standard deviation by three and subtracting this result from the mean. (Answer: F) Control charts can never be used by the laboratory for trend analysis. (Answer: F) |

10.  The following may be used to monitor a laboratory’s test results for validity: (Answer: D)

| A. Certified reference material | B. Proficiency testing | C. Retesting | D. All the choices. |
| --- | --- | --- | --- |

11.	When quality control data is found to be outside of the laboratory’s predefined criteria: (Answer: B)

| A. Quality control is repeated until acceptable results are achieved. | B. Planned action is taken to correct the problem and prevent incorrect results from being reported. | C. Instruments used in the analyses are to have preventive maintenance performed. | D. All results from that batch of samples are to be destroyed. |
| --- | --- | --- | --- |

12.	QC samples used for control charting can be: (Answer: D)

| A. Reference material or certified reference material. | B. Old PT samples (with associated statistics). | C. A sample characterized by the laboratory. | D. All of the choices. |
| --- | --- | --- | --- |

13.	A control chart contains all of the following except: (Answer: A)

| A. Exponential value of the standard deviation | B. Mean. | C. Upper control level. | D. Plotted data. |
| --- | --- | --- | --- |

14.	Control chart statistics are reviewed: (Answer: C)

| A. Only by the laboratory’s QA manager. | B. Annually. | C. At intervals set by the laboratory’s QMS. | D. Daily |
| --- | --- | --- | --- |

15.	A laboratory’s process is considered (statistical) out of control when: (Answer: B)

| A. When one quality control point falls outside of the upper or lower control limits | B. Indicated by the laboratory’s QMS | C. One quality control point falls outside of the upper or lower warning limits. | D. A series of quality control points fall above or below the mean. |
| --- | --- | --- | --- |

16.  Qualitative analysis (presence/absence): (Answer: C)

| A. Never need to be evaluated for trends. | B. Never have values that can be control charted. | C. May have control charts associated with them per the laboratory’s QMS. | D. Always have skewed control charts. |
| --- | --- | --- | --- |