QMS Quick Learning Activity

QMS Quick Learning Activity

# Traceability

# Quiz-Answer Key

Passing score = 80%   [Answers in RED]

Select the correct answer.

| 1. | T | F | ISO/IEC 17025 does not require laboratories to have calibration systems in place. (Answer: F) |
| --- | --- | --- | --- |
| 2. | T | F | ISO Guide 34 specifies requirements for reference material producers. (Answer: T) |
| 3. | T | F | Calibration laboratories do not need to establish traceability. (Answer: F) |
| 4. | T | F | Calibration laboratories must issue certificates that include measurement uncertainty. (Answer: T) |
| 5. | T | F | Traceability does not apply to instruments and equipment. (Answer: F) |
| 6. | T | F | Traceability can be referred to as a “calibration hierarchy”. (Answer: T) |
| 7. | T | F | Calibration is the comparison of two measurement devices or systems, one of known uncertainty (reference standard) and one of unknown uncertainty (test equipment). (Answer: T) |

- Reference Standards include: (Answer: A)

| A. Hard and non-consumable items | B. Liquid standards | C. Consumables | D. Proficiency testing materials |
| --- | --- | --- | --- |

- Reference Materials include:  (Answer: C)

| A. Hard and non-consumable items | B. NIST traceable thermometers | C. Consumables | D. Weights |
| --- | --- | --- | --- |

- Reference Standards should be: (Answer: D)

| A. Traced to SI units | B. Used for calibration only to maintain accuracy and integrity | C. Calibrated by an ISO/IEC 17025 accredited body that can provide traceability | D. B and C |
| --- | --- | --- | --- |

- Reference Materials should be: (Answer: A)

| A. Traced to SI units | B. Used for calibration only to maintain accuracy and integrity | C. Calibrated by an ISO/IEC 17025 accredited body that can provide traceability | D. Made by the laboratory in-house for best results |
| --- | --- | --- | --- |

- Intermediate checks need defined procedures and schedules for carrying out checks to maintain confidence in the calibration status of: (Answer: D)

| A. Reference Standards and Materials | B. Primary and Working Standards | C. Transfer Standards | D. All of the choices |
| --- | --- | --- | --- |