QMS Quick Learning Activity

QMS Quick Learning Activity

# Traceability

# Quiz

Passing score = 80%

Select the correct answer.

| 1. | T | F | ISO/IEC 17025 does not require laboratories to have calibration systems in place. |
| --- | --- | --- | --- |
| 2. | T | F | ISO Guide 34 specifies requirements for reference material producers. |
| 3. | T | F | Calibration laboratories do not need to establish traceability. |
| 4. | T | F | Calibration laboratories must issue certificates that include measurement uncertainty. |
| 5. | T | F | Traceability does not apply to instruments and equipment. |
| 6. | T | F | Traceability can be referred to as a “calibration hierarchy”. |
| 7. | T | F | Calibration is the comparison of two measurement devices or systems, one of known uncertainty (reference standard) and one of unknown uncertainty (test equipment). |

- Reference Standards include:

| A. Hard and non-consumable items | B. Liquid standards | C. Consumables | D. Proficiency testing materials |
| --- | --- | --- | --- |

- Reference Materials include:

| A. Hard and non-consumable items | B. Liquid standards | C. Consumable | D. A & B |
| --- | --- | --- | --- |

- Reference Standards should be:

| A. Traced to SI units | B. Provided by a competent supplier | C. Calibrated by an ISO/IEC 17025 accredited body that can provide traceability | D. A & B |
| --- | --- | --- | --- |

- Reference Materials should be:

| A. Traced to SI units | B. Provided by a competent supplier | C. Calibrated by an ISO/IEC 17025 accredited body that can provide traceability | D. A, B & C |
| --- | --- | --- | --- |

- Intermediate checks need defined procedures and schedules for carrying out checks to maintain confidence in the calibration status of:

| A. Reference Standards and Materials | B. Primary and Working Standards | C. Transfer Standards | D. All of the choices |
| --- | --- | --- | --- |