QMS Quick Learning Activity

QMS Quick Learning Activity

# Equipment and Calibration

# Quiz-Answer Key

Passing score = 80%   [Answers in RED]

Select the correct answer.

| 1. | T | F | Calibration is the comparison of two measurement devices or systems (one of known uncertainty (your standard) and one of unknown uncertainty (your test equipment). (Answer: T) |
| --- | --- | --- | --- |
| 2. | T | F | Traceability is the property of a measurement result whereby the result can be related to a reference through a documented unbroken chain of calibrations, each contributing to the measurement uncertainty. (Answer: T) |
| 3. | T | F | Electronic calculations and data transfers are not subject to appropriate checks in a systematic manner. (Answer: F) |
| 4. | T | F | Commercial software (e.g., word processing, database, statistical programming) is considered sufficiently validated. (Answer: T) |
| 5. | T | F | Only certain measurement and testing equipment have a significant effect on the validity and accuracy of test results. (Answer: F) |
| 6. | T | F | Equipment sent out of the laboratory for calibration is ready to go when received back in the laboratory. (Answer: F) |

- There need to be defined procedures and schedules for carrying out checks needed to maintain confidence in the calibration status of:  (Answer: D)

| A. Reference standards | B. Primary, working, and transfer standards | C. Reference materials | D. A, B & C |
| --- | --- | --- | --- |

- The following support equipment requires calibration except (Answer: C)

| A. Incubators | B. Waterbaths | C. Stir plates | D. Thermometers |
| --- | --- | --- | --- |

- Equipment that has been mishandled, gives suspect results, is defective, or gives suspect results shall be:.? (Answer: D)

| A. Taken out of service | B. Clearly marked as being out of service | C. Decommissioned and discarded immediately | D. A & B only |
| --- | --- | --- | --- |

10.  All the following are correct regarding calibrated equipment except:  (Answer: D)

| A. Can be labelled to indicate calibration status | B. Can be coded to indicated calibration status | C. Can be otherwise identified to indicate the status of the calibration | D. All equipment in the laboratory is assumed to be calibrated so no action or labelling is required |
| --- | --- | --- | --- |